Simple DC Circuits – 16/09/2024 – Kait Mounce, Christopher Hamilton, Garrett Haldrup

Rita Hollings Science Center Room 371, 58 Coming St, Charleston, SC 29401

Professor Dr. Dileep Karanth, Phys 112-01L

ABSTRACT

This lab we set out to understand Ohm’s Law and how resistors in series and parallel act. We learned about Ohm’s Law and how to calculate the equivalent resistance of a series and parallel circuit. Using this we were able to see and learn that for a series circuit the current across each resistor stays the same, but voltage across each one is different. For a parallel circuit we learned that the current changes for each resistor while voltage remains the same. This explains what we saw with the light bulbs getting dimmer for each one added in series and staying the same for ones in parallel. We were able to learn this by building these simple circuits with resistors and measuring the voltage and current with a multimeter

INTRO/THEORY

This lab is over simple dc circuits and how putting resistors in parallel or series effects the current and voltage across the resistors. To do this you can use Ohm’s Law to help see what values for voltage and current should be expected when you measure. Ohm’s Law is V = IR where V is voltage, I is current, and R is resistance. Then for calculating the resistance of a circuit you need to get the equivalent resistance for the whole circuit as one resistor. For resistors in series this is easy where Req = R1 + R2 + … Rn For resistors in parallel it is a little more complex where first you calculate the inverse of the equivalent resistance by summing the inverse of all resistors in parallel. 1Req=1R1+ 1R2+…+ 1Rn. Using these simple equations, it is possible to calculate the total current of a circuit or voltage drop of a circuit and for each resistor in that circuit.

DESCRIPTION

To set up for this lab we first plugged in our power supply and set it to 5 volts. Then we took our wires with alligator clips and for the first setup we took two lightbulb sockets and wired them in series. After this experiment we then connected the three resistors in parallel with the wires and to measure voltage put the multimeter in parallel with the resistor and for current put the multimeter in series with the resistor. This is also the same setup of parallel resistors.

PROCEDURE

For this lab we first started out by setting our power supply to 5 volts, and then we set up a simple series circuit with light bulbs. We then add more light bulbs to the circuit in series with the others and recorded our observations of their brightness. We then deconstructed our circuit with lightbulbs and built one that uses our three resistors in series. We then calculated what our voltage drop should be across each resistor using ohms law, then we used a multimeter set to be a voltmeter to measure the voltage drop across each resistor. We then also did the same for the current across each resistor by changing our multimeter to an ammeter. Then for parallel circuits we did the same steps as before where we first made it with light bulbs and observed what adding more does. Then remaking the circuit with the resistors and calculation voltage drop and current using ohms law. Then using a multimeter to compare our calculations.

RESULTS

Part I – Observations

| Circuit Type | Observation |
| --- | --- |
| Series | As we added more light bulbs in series, we saw the brightness of all bulbs in the circuit decreased, but all remained equal to each other. |
| Parallel | As we added more light bulbs in parallel, we saw the brightness of all bulbs in the circuit remain the same and equal to each other. |

Table 1.1

This table contains our observations when we add more light bulbs into series and parallel circuits.

Part II – Series Circuit

Current

|  | Ia(mA)(1000Ω) | Ib(mA)(2200Ω) | Ic(mA)(3300Ω) | It(mA) (total) |
| --- | --- | --- | --- | --- |
| Measured | 0.75 | 0.77 | 0.77 | 0.77 |
| Calculated | 0.77 | 0.77 | 0.77 | 0.77 |
| Error % | 2.6 | 0 | 0 | 0 |

Table 1.1

This table shows our calculations for the current across each resistor in a series circuit and the measured value with experimental error for the difference between our measured and calculated values.

Voltage

|  | Va(V)(1000Ω) | Vb(V)(2200Ω) | Vc(V)(3300Ω) | Vt(V) (total) |
| --- | --- | --- | --- | --- |
| Measured | 0.788 | 1.700 | 2.533 | 4.999 |
| Calculated | 0.769 | 1.692 | 2.533 | 5 |
| Error % | 2.47 | 0.47 | 0.197 | 0.02 |

Table 1.2

This table shows our calculations for the voltage drop across each resistor in a series circuit and the measured value with experimental error for the difference between our measured and calculated values.

Part III – Parallel Circuit

Current

|  | Ia(mA)(1000Ω) | Ib(mA)(2200Ω) | Ic(mA)(3300Ω) | It(mA) (total) |
| --- | --- | --- | --- | --- |
| Measured | 4.90 | 2.31 | 1.52 | 8.63 |
| Calculated | 5.00 | 2.27 | 1.52 | 8.79 |
| Error % | 2 | 1.76 | 0 | 1.82 |

Table 2.1

This table shows our calculations for the current across each resistor in a parallel circuit and the measured value with experimental error for the difference between our measured and calculated values.

Voltage

|  | Va(V)(1000Ω) | Vb(V)(2200Ω) | Vc(V)(3300Ω) | Vt(V) (total) |
| --- | --- | --- | --- | --- |
| Measured | 4.99 | 4.99 | 4.99 | 4.99 |
| Calculated | 5 | 5 | 5 | 5 |
| Error % | 0.02 | 0.02 | 0.02 | 0.02 |

Table 2.2

This table shows our calculations for the voltage drop across each resistor in a parallel circuit and the measured value with experimental error for the difference between our measured and calculated values.

CONCLUSIONS

For circuits with resistors in series our measurements and calculations say that the current across each resistor and the whole circuit will remain the same, but each resistor will experience a different voltage to keep ohms law valid. This voltage that each resistor experiences will add up to the total voltage difference of the system.

For circuits with resistors in parallel we see the measurements and calculations say that voltage across each resistor will be the same, but each resistor will have a different current across it. The current across each resistor will add up to the total current of the system.